- International AC input range

- High efficiency,low working temperature

- Short circuit,overload,over-current protection

- 100% fully load burn-in test

- Built-in EMI filter, low ripple noise

- Low cost, high reliability

- CE and Rohs approval,2 years warranty

- OEM and ODM is welcome

| Model | S-50-5 |  |
| --- | --- | --- |
| AC input voltage range | 100-240VAC |  |
| Input current | 0.7A/115V 0.35A/230V |  |
| AC impulse current | Cold-start current 13A/115V 26A/230V |  |
| DC output voltage | 5V |  |
| DC output power | 50W |  |
| Rated output current | 10A |  |
| Output voltage error | ±1% |  |
| Efficiency | 80% |  |
| Wave and noise | 300mVp-p |  |
| Inlet stability | ±0.5% |  |
| Load stability | ±0.5% |  |
| Leakage current | <0.5mA/240VAC |  |
| Overload protection | 105% - 150% rated power, automatically recovery |  |
| Vibration | 10500Hz,2G 10min./1cycle,Period for 60min,Each axes |  |
| Cooling method | Natural cooling air convection |  |
| Working temperature and humidity | - 20 ° ~ 60 ° c / 20% ~ 90% RH (no frost) |  |
| Store temperature and humidity | - 40 ° ~ 85 ° c / 10% ~ 95% RH (no frost) |  |
| Safety standards | CE(EMC+LVD),Rohs |  |
| Dimension | 110x78x35mm |  |
| Weight | 0.23kgs |  |

1.LED switching power supply using pulse width modulation (PWM) switching technology designed to ensure power supply stability and high efficiency.

2. Product interference, through the EMC testing, ripple ≤ 50MV.

3. Efficiency above 80%, temperature ≤ 25 °C.

4.LED switching power supply with overvoltage protection, short circuit protection, overload protection.

5. Constant current output to ensure the stability of the power, so that LED lighting effects better and live longer.

6. Products have the anti-interference ensures long-life LED strips, reducing the LED lighting decline.